NEXORA TECHNOLOGIES LTD

Enterprise Network Design and Implementation

Dual-Site Network Infrastructure — Lagos HQ & Ikeja Branch

| Document Details Document Title: Enterprise Network Design Client: Nexora Technologies Ltd Industry: Financial Technology (Fintech) Version: 1.0 Date: March 2026 Status: Final | Scope Summary Sites: 2 (Lagos HQ + Ikeja Branch) Devices: 15 Cisco routers and switches VLANs: 6 (VLAN 10, 20, 30, 40, 50, 60) WAN: IPsec over GRE (AES-256) Platform: GNS3 simulation environment Engineer: Network Deployment Engineer |
| --- | --- |

# 1. Executive Summary

Nexora Technologies Ltd engaged this project to design and implement a complete enterprise network infrastructure connecting their Lagos Island headquarters and Ikeja branch office. The network was required to support six internal departments across both sites, provide encrypted and redundant inter-site connectivity, and deliver a fault-tolerant design that could withstand single points of failure without service interruption.

The solution delivers a fully routed, dual-site enterprise network built on industry-standard protocols. All inter-site traffic traverses an IPsec-encrypted GRE tunnel, protecting sensitive financial data in transit. Redundant gateways ensure no single device failure disrupts connectivity for any department. Centralised monitoring through syslog and NTP provides the operational visibility required for a financial services environment.

Key Outcomes Delivered Secure encrypted site-to-site connectivity between Lagos HQ and Ikeja Branch Full redundancy at Layer 2 and Layer 3 — no single point of failure for any VLAN Automated failover via IP SLA object tracking on both edge routers Centralised syslog collection and NTP synchronisation across all 15 devices Hardened device access with consistent security baseline across entire infrastructure Complete interface documentation and professional naming convention throughout

# 2. Client and Project Background

## 2.1 Organisation Profile

| Field | Details |
| --- | --- |
| Company Name | Nexora Technologies Ltd |
| Industry | Financial Technology (Fintech) |
| Employee Count | Approximately 200 staff |
| Headquarters | Lagos Island, Lagos State |
| Branch Office | Ikeja, Lagos State |
| Primary Requirement | Secure, redundant dual-site network infrastructure |
| Simulation Platform | EVE-NG (Cisco IOS and IOS-XE devices) |

## 2.2 Department Mapping

Each VLAN maps to a specific department at its respective site. This segmentation isolates department traffic, enables per-VLAN security policies, and provides a clear structure for future growth.

| VLAN | Subnet | Department | Site | Sensitivity |
| --- | --- | --- | --- | --- |
| VLAN 10 | 10.1.1.0/24 | Finance and Accounts | Lagos HQ | High |
| VLAN 20 | 20.1.1.0/24 | Engineering and Development | Lagos HQ | Medium |
| VLAN 30 | 30.1.1.0/24 | Sales and Business Development | Lagos HQ | Medium |
| VLAN 40 | 40.1.1.0/24 | Management and Executive | Lagos HQ | High |
| VLAN 50 | 50.1.1.0/24 | IT Infrastructure and Operations | Ikeja Branch | High |
| VLAN 60 | 60.1.1.0/24 | Customer Support and Service Desk | Ikeja Branch | Medium |

# 3. Network Architecture and Design

## 3.1 High-Level Architecture

The network follows a three-tier hierarchical design at each site — access, distribution, and core — with the distribution and core layers collapsed into a single tier for efficiency at this scale. Both sites connect to a simulated ISP transit router representing the internet handoff point.

| Tier | HQ Devices | IKY Devices | Role |
| --- | --- | --- | --- |
| Edge | NXR-HQ-EDGE-R1 | NXR-IKY-EDGE-R1 | NAT, IPsec VPN, default routing, IP SLA |
| Core/Distribution | NXR-HQ-CORE-SW1, SW2 | NXR-IKY-CORE-SW1 to SW4 | L3 routing, HSRP, STP root, EtherChannel |
| Access | NXR-HQ-ACC-SW1 to SW4 | NXR-IKY-ACC-SW1, SW2 | VLAN access, PortFast, BPDU Guard |
| WAN/ISP | ISP-TRANSIT-R1 (simulated ISP) | - | Internet transit simulation, NTP master |

## 3.2 Device Inventory and Naming Convention

All devices follow the naming convention: [CLIENT]-[SITE]-[ROLE]-[TYPE][NUMBER]. This ensures any engineer can identify a device's function and location from its hostname alone without consulting any additional documentation.

| Hostname | Role | Platform | Site |
| --- | --- | --- | --- |
| ISP-TRANSIT-R1 | Simulated ISP transit router | Cisco CSR1000V (IOS-XE 17.3) | ISP |
| NXR-HQ-EDGE-R1 | HQ edge router — NAT, IPsec, default routing | Cisco CSR1000V (IOS-XE 17.3) | Lagos HQ |
| NXR-HQ-CORE-SW1 | HQ core/distribution switch — primary | Cisco IOSv (15.2) | Lagos HQ |
| NXR-HQ-CORE-SW2 | HQ core/distribution switch — secondary | Cisco IOSv (15.2) | Lagos HQ |
| NXR-HQ-ACC-SW1 | HQ access switch — Finance (VLAN 10) | Cisco IOSv (15.2) | Lagos HQ |
| NXR-HQ-ACC-SW2 | HQ access switch — Engineering (VLAN 20) | Cisco IOSv (15.2) | Lagos HQ |
| NXR-HQ-ACC-SW3 | HQ access switch — Sales (VLAN 30) | Cisco IOSv (15.2) | Lagos HQ |
| NXR-HQ-ACC-SW4 | HQ access switch — Management (VLAN 40) | Cisco IOSv (15.2) | Lagos HQ |
| NXR-IKY-EDGE-R1 | IKY edge router — NAT, IPsec, default routing | Cisco CSR1000V (IOS-XE 17.3) | Ikeja Branch |
| NXR-IKY-CORE-SW1 | IKY core switch — primary, VLAN 50 root | Cisco IOSv (15.2) | Ikeja Branch |
| NXR-IKY-CORE-SW2 | IKY core switch — secondary, VLAN 60 root | Cisco IOSv (15.2) | Ikeja Branch |
| NXR-IKY-CORE-SW3 | IKY core switch — tertiary redundancy | Cisco IOSv (15.2) | Ikeja Branch |
| NXR-IKY-CORE-SW4 | IKY core switch — quaternary redundancy | Cisco IOSv (15.2) | Ikeja Branch |
| NXR-IKY-ACC-SW1 | IKY access switch — IT Infrastructure (VLAN 50) | Cisco IOSv (15.2) | Ikeja Branch |
| NXR-IKY-ACC-SW2 | IKY access switch — Customer Support (VLAN 60) | Cisco IOSv (15.2) | Ikeja Branch |

# 4. IP Addressing

## 4.1 VLAN Subnets — End User Networks

| VLAN | Network | Subnet Mask | Gateway (Physical) | Virtual Gateway (HSRP) | Department |
| --- | --- | --- | --- | --- | --- |
| 10 | 10.1.1.0/24 | 255.255.255.0 | 10.1.1.254 (SW1), 10.1.1.253 (SW2) | 10.1.1.100 | Finance and Accounts |
| 20 | 20.1.1.0/24 | 255.255.255.0 | 20.1.1.254 (SW1), 20.1.1.253 (SW2) | 20.1.1.100 | Engineering and Dev |
| 30 | 30.1.1.0/24 | 255.255.255.0 | 30.1.1.254 (SW1), 30.1.1.253 (SW2) | 30.1.1.100 | Sales and BizDev |
| 40 | 40.1.1.0/24 | 255.255.255.0 | 40.1.1.254 (SW1), 40.1.1.253 (SW2) | 40.1.1.100 | Management |
| 50 | 50.1.1.0/24 | 255.255.255.0 | 50.1.1.254 (SW1) to 50.1.1.251 (SW4) | 50.1.1.100 | IT Infrastructure |
| 60 | 60.1.1.0/24 | 255.255.255.0 | 60.1.1.254 (SW1) to 60.1.1.251 (SW4) | 60.1.1.100 | Customer Support |

## 4.2 Infrastructure Links

| Network | Subnet Mask | Connects | Purpose |
| --- | --- | --- | --- |
| 1.1.1.0/30 | 255.255.255.252 | ISP-TRANSIT-R1 — NXR-HQ-EDGE-R1 | HQ WAN transit link |
| 2.2.2.0/30 | 255.255.255.252 | ISP-TRANSIT-R1 — NXR-IKY-EDGE-R1 | IKY WAN transit link |
| 12.1.1.0/24 | 255.255.255.0 | NXR-HQ-EDGE-R1 — NXR-HQ-CORE-SW1 | HQ edge to core link 1 |
| 13.1.1.0/24 | 255.255.255.0 | NXR-HQ-EDGE-R1 — NXR-HQ-CORE-SW2 | HQ edge to core link 2 |
| 11.1.1.0/24 | 255.255.255.0 | NXR-HQ-CORE-SW1 — NXR-HQ-CORE-SW2 | HQ core EtherChannel (Po10) |
| 14.1.1.0/24 | 255.255.255.0 | NXR-IKY-EDGE-R1 — NXR-IKY-CORE-SW1 | IKY edge to core link 1 |
| 15.1.1.0/24 | 255.255.255.0 | NXR-IKY-EDGE-R1 — NXR-IKY-CORE-SW2 | IKY edge to core link 2 |
| 16.1.1.0/24 | 255.255.255.0 | NXR-IKY-EDGE-R1 — NXR-IKY-CORE-SW3 | IKY edge to core link 3 |
| 17.1.1.0/24 | 255.255.255.0 | NXR-IKY-EDGE-R1 — NXR-IKY-CORE-SW4 | IKY edge to core link 4 |
| 18.1.1.0/24 | 255.255.255.0 | NXR-IKY-CORE-SW1 — NXR-IKY-CORE-SW2 | IKY core EtherChannel (Po10) |
| 19.1.1.0/24 | 255.255.255.0 | NXR-IKY-CORE-SW2 — NXR-IKY-CORE-SW3 | IKY core EtherChannel (Po20) |
| 20.1.1.0/24 | 255.255.255.0 | NXR-IKY-CORE-SW3 — NXR-IKY-CORE-SW4 | IKY core EtherChannel (Po30) |
| 192.168.10.0/24 | 255.255.255.0 | NXR-HQ-EDGE-R1 — NXR-IKY-EDGE-R1 | GRE/IPsec tunnel (EIGRP) |
| 8.8.8.8/24 | 255.255.255.0 | ISP-TRANSIT-R1 Loopback1 | Simulated internet/DNS target |

# 5. Protocol Design and Justification

## 5.1 Routing — OSPF

OSPF (Open Shortest Path First) was selected as the interior gateway protocol for both sites. OSPF is a link-state protocol that maintains a complete map of the network topology, enabling fast convergence after any link or device failure. All infrastructure links and VLAN subnets are advertised within OSPF Area 0 (the backbone area).

VLAN interfaces (SVIs) on all core and distribution switches are configured as passive interfaces. This prevents OSPF Hello packets from being sent into end-user VLANs where no OSPF routers exist, reducing unnecessary traffic and preventing rogue adjacency formation. The subnets are still advertised into OSPF — only the Hello process is suppressed.

## 5.2 Routing — EIGRP Over GRE Tunnel

EIGRP (Enhanced Interior Gateway Routing Protocol) runs exclusively over the GRE tunnel between NXR-HQ-EDGE-R1 and NXR-IKY-EDGE-R1. EIGRP was chosen for the tunnel because it supports multicast and maintains neighbour relationships, making it ideal for carrying routing updates over a point-to-point logical link.

IPsec alone cannot carry routing protocols because it only handles unicast traffic. The GRE tunnel provides the multicast capability required for EIGRP neighbour discovery, and IPsec secures the GRE envelope. This combination — GRE for encapsulation and EIGRP for routing, secured by IPsec — is a well-established enterprise WAN design pattern.

## 5.3 Route Redistribution

Because OSPF and EIGRP operate as separate routing domains, redistribution was configured on both edge routers to exchange routes between them. Each edge router runs both protocols simultaneously and acts as the translation point between the two domains.

| Direction | Command | Where Configured | Effect |
| --- | --- | --- | --- |
| EIGRP into OSPF | redistribute eigrp 10 subnets | Both edge routers, under router ospf 1 | VLAN routes from the remote site appear as OSPF External Type 2 (E2) in the local site |
| OSPF into EIGRP | redistribute ospf 1 metric 10000 100 255 1 1500 | Both edge routers, under router eigrp 10 | Local VLAN routes are injected into EIGRP and carried across the tunnel to the remote site |
| Default route | default-information originate | Both edge routers, under router ospf 1 | Internet default route is propagated to all OSPF neighbours, enabling internet access from all VLANs |

The subnets keyword on the OSPF redistribution statement is critical. Without it, OSPF only accepts classful network boundaries and ignores the specific /24 subnets. This was identified as a key configuration requirement during implementation.

## 5.4 Gateway Redundancy — HSRP

HSRP (Hot Standby Router Protocol) provides redundant default gateways for all end-user VLANs. Each VLAN has a virtual IP address that end devices use as their default gateway. One switch actively forwards traffic for that virtual IP while others stand by.

Priorities were deliberately configured to distribute active gateway responsibility across switches, providing load balancing and ensuring that the failure of any single switch does not affect more than half of the VLANs. The preempt keyword was configured on active switches to ensure they reclaim the active role after recovering from a failure.

| VLAN | HSRP Active | HSRP Standby | Priority (Active) | Virtual IP |
| --- | --- | --- | --- | --- |
| 10 | NXR-HQ-CORE-SW1 | NXR-HQ-CORE-SW2 | 110 | 10.1.1.100 |
| 20 | NXR-HQ-CORE-SW1 | NXR-HQ-CORE-SW2 | 110 | 20.1.1.100 |
| 30 | NXR-HQ-CORE-SW2 | NXR-HQ-CORE-SW1 | 110 | 30.1.1.100 |
| 40 | NXR-HQ-CORE-SW2 | NXR-HQ-CORE-SW1 | 110 | 40.1.1.100 |
| 50 | NXR-IKY-CORE-SW1 | NXR-IKY-CORE-SW2 | 110 | 50.1.1.100 |
| 60 | NXR-IKY-CORE-SW2 | NXR-IKY-CORE-SW1 | 110 | 60.1.1.100 |

## 5.5 Spanning Tree — STP with HSRP Alignment

Spanning Tree Protocol (PVST+) prevents Layer 2 loops across the redundant access-to-core uplinks. A critical design principle was applied throughout: the STP root bridge for each VLAN is always the same switch as the HSRP active gateway for that VLAN.

Without this alignment, traffic from access switches would follow the STP-preferred path to the root bridge, then travel an additional hop to reach the HSRP active gateway — creating unnecessary traffic and consuming bandwidth. Aligning STP root with HSRP active ensures the optimal path is used for all traffic.

| VLAN | STP Root (Primary) | STP Root (Secondary) | Priority |
| --- | --- | --- | --- |
| 10 | NXR-HQ-CORE-SW1 | NXR-HQ-CORE-SW2 | 4096 / 8192 |
| 20 | NXR-HQ-CORE-SW1 | NXR-HQ-CORE-SW2 | 4096 / 8192 |
| 30 | NXR-HQ-CORE-SW2 | NXR-HQ-CORE-SW1 | 4096 / 8192 |
| 40 | NXR-HQ-CORE-SW2 | NXR-HQ-CORE-SW1 | 4096 / 8192 |
| 50 | NXR-IKY-CORE-SW1 | NXR-IKY-CORE-SW2 | 4096 / 8192 |
| 60 | NXR-IKY-CORE-SW2 | NXR-IKY-CORE-SW1 | 4096 / 8192 |

STP priorities are set in multiples of 4096 as required by IOS. This is a hardware constraint — the lower 12 bits of the priority field are reserved for the VLAN ID system extension, leaving only 16 possible values for the configurable portion.

## 5.6 EtherChannel

EtherChannel bundles multiple physical links between core switches into a single logical link, providing both increased bandwidth and link redundancy. If one physical member link fails, traffic redistributes across the remaining links automatically with no STP reconvergence and no downtime.

| Port Channel | Connects | Member Links | Mode | IP Address |
| --- | --- | --- | --- | --- |
| Po10 (HQ) | NXR-HQ-CORE-SW1 to SW2 | Gi0/1, Gi0/2, Gi0/3, Gi1/0 | Static (mode on) | 11.1.1.0/24 |
| Po10 (IKY) | NXR-IKY-CORE-SW1 to SW2 | Gi0/0, Gi0/1 | PAGP (desirable/auto) | 18.1.1.0/24 |
| Po20 (IKY) | NXR-IKY-CORE-SW2 to SW3 | Gi0/2, Gi0/3 | PAGP (desirable/auto) | 19.1.1.0/24 |
| Po30 (IKY) | NXR-IKY-CORE-SW3 to SW4 | Gi0/2, Gi0/3 | LACP (active/passive) | 20.1.1.0/24 |

# 6. Security Implementation

## 6.1 IPsec over GRE — Site-to-Site VPN

All traffic traversing the WAN between the two sites is protected by IPsec encryption. The tunnel uses GRE encapsulation to carry routing protocols and multicast, with IPsec applied in transport mode to encrypt the GRE payload.

| Parameter | Value | Justification |
| --- | --- | --- |
| Encryption | AES-256 | Industry standard. 256-bit key provides strong confidentiality for financial data |
| Hashing | SHA-256 | Cryptographically secure integrity verification |
| Authentication | Pre-shared key | Appropriate for two-endpoint VPN. Certificate-based auth recommended for larger deployments |
| DH Group | Group 14 (2048-bit) | Minimum recommended group. Groups 1 and 2 are deprecated and insecure |
| Phase 1 Lifetime | 86400 seconds (24 hours) | Standard enterprise renegotiation interval |
| ESP Mode | Transport mode | Correct mode when IPsec protects a GRE tunnel — preserves the GRE outer header |
| NAT Exclusion | ACL 110 deny between tunnel endpoints | Prevents NAT from intercepting IKE negotiation packets on UDP 500 |

## 6.2 Access Port Hardening — PortFast and BPDU Guard

All switch ports connected to end devices are configured with PortFast and BPDU Guard. PortFast allows the port to transition directly to forwarding state, bypassing the 30-second STP delay that would otherwise prevent a device from communicating after connecting.

BPDU Guard protects against rogue switches being connected to access ports. If a BPDU is received on a PortFast-enabled port, the port is immediately placed into err-disabled state. This prevents an unauthorised switch from influencing the STP topology and potentially disrupting the network.

Important: PortFast and BPDU Guard are always deployed together PortFast alone is dangerous — it skips STP checks but provides no protection against rogue switches BPDU Guard is the enforcement mechanism that makes PortFast safe to deploy These features are configured per-interface on all access-facing ports across all access switches Trunk uplinks to core switches must never have PortFast or BPDU Guard applied

## 6.3 Device Access Security

A consistent security baseline was applied to every managed device in the network. The following controls are in place on all 15 devices:

| Control | Configuration | Purpose |
| --- | --- | --- |
| Password encryption | service password-encryption | Prevents plaintext passwords in running configuration |
| Local authentication | username netadmin privilege 15 secret | Type 9 hashed credentials for device access |
| SSH only | transport input ssh / ip ssh version 2 | Telnet disabled. SSHv2 only — SSHv1 is deprecated and insecure |
| Session timeout | exec-timeout 10 0 | Idle sessions automatically disconnected after 10 minutes |
| DNS lookup disabled | no ip domain-lookup | Prevents router hanging on mistyped commands during operation |
| Legal warning banner | banner motd | Establishes legal notice of monitoring and restricted access |
| Synchronous logging | logging synchronous | Console messages do not interrupt active command entry |

# 7. Operations and Monitoring

## 7.1 NTP — Time Synchronisation

Accurate and consistent timestamps across all devices are essential for security auditing, incident investigation, and log correlation. A hierarchical NTP architecture was implemented with the simulated ISP router as the authoritative time source.

| Stratum | Device | NTP Source | Role |
| --- | --- | --- | --- |
| Stratum 1 | ISP-TRANSIT-R1 | ntp master 1 (self-authoritative) | Lab NTP master — simulates external time source |
| Stratum 2 | NXR-HQ-EDGE-R1 | 8.8.8.8 (ISP loopback) | HQ time source for core switches |
| Stratum 2 | NXR-IKY-EDGE-R1 | 8.8.8.8 (ISP loopback) | IKY time source for core switches |
| Stratum 3 | NXR-HQ-CORE-SW1 | 12.1.1.2 (Edge R1 Gi1) | HQ distribution time source |
| Stratum 3 | NXR-HQ-CORE-SW2 | 13.1.1.2 (Edge R1 Gi2) | HQ distribution time source |
| Stratum 3 | NXR-IKY-CORE-SW1-4 | 14-17.1.1.2 (Edge R2) | IKY distribution time source |
| Stratum 4 | All access switches | VLAN virtual gateway IP | Access layer time synchronisation |

All devices are set to West Africa Time (WAT, UTC+1) using clock timezone WAT 1. Timestamps include millisecond precision for precise event correlation.

## 7.2 Syslog — Centralised Logging

All network devices send log messages to the IT Operations Linux server at 50.1.1.10 (VLAN 50 — IT Infrastructure). Logging is configured at informational severity (Level 6), capturing all meaningful operational events without generating excessive volume from debugging traffic.

| Severity Level | Events Captured | Relevance |
| --- | --- | --- |
| 0-3 (Emergency to Error) | System failures, critical errors | Immediate action required |
| 4 (Warning) | Interface state changes, authentication failures | Requires investigation |
| 5 (Notification) | OSPF/EIGRP neighbour changes, HSRP state transitions | Routing and redundancy events |
| 6 (Informational) | IPsec SA establishment, NTP sync, config changes | Operational audit trail |
| 7 (Debug) | Packet-level events | Not captured — excessive volume |

Centralised syslog enables rapid incident investigation. When an event occurs, timestamps from all devices can be correlated in a single log view to build an accurate timeline. Without centralised logging, an engineer would need to SSH into each device individually and risk missing events from devices with overwritten local buffers.

## 7.3 IP SLA — Automated Failover Monitoring

IP SLA probes continuously test WAN path availability on both edge routers. Each probe sends an ICMP echo to the ISP next-hop every 5 seconds. The result is tied to a tracking object that controls the default route — if the probe fails, the static default route is automatically withdrawn from the routing table.

| Parameter | NXR-HQ-EDGE-R1 | NXR-IKY-EDGE-R1 |
| --- | --- | --- |
| Probe target | 1.1.1.2 (ISP Gi2) | 2.2.2.1 (ISP Gi1) |
| Source interface | GigabitEthernet3 | GigabitEthernet5 |
| Probe frequency | Every 5 seconds | Every 5 seconds |
| Timeout threshold | 2000ms | 2000ms |
| Failure threshold | 1000ms RTT | 1000ms RTT |
| Tracked route | 0.0.0.0/0 via 1.1.1.2 | 0.0.0.0/0 via 2.2.2.1 |
| Failover action | Route removed from table | Route removed from table |

Without IP SLA tracking, a failed WAN link would leave the static default route in the routing table, causing all internet-bound traffic to enter a black hole. IP SLA eliminates this risk through continuous automated verification.

# 8. Implementation Challenges and Lessons Learned

This section documents significant issues encountered during implementation. These are recorded not as failures but as valuable troubleshooting experiences that inform future deployments.

## 8.1 NAT Conflicting with IPsec IKE Negotiation

Problem: After configuring IPsec, Phase 1 negotiation was stuck at MM_KEY_EXCH and never reached QM_IDLE. The show ip nat translations output revealed that UDP port 500 (ISAKMP) packets were being translated before IPsec could process them.

Root Cause: The original NAT ACL used permit any, causing NAT to translate all outbound traffic including IKE packets. When the source port was changed by NAT, the receiving router rejected the packets as they did not match the expected peer configuration.

Resolution: The NAT ACL was replaced with an extended ACL (numbered in the 100-199 range) that explicitly denied translation between the two tunnel endpoint IPs before permitting everything else. This allowed IKE packets to reach the peer with their original source addresses intact.

Key Lesson When IPsec and NAT coexist on the same router, NAT must explicitly exclude tunnel endpoint traffic. Standard ACLs (1-99) cannot be used for this purpose — they only match source IP. Extended ACLs (100-199) are required to match both source and destination. Symptom of this issue: MM_KEY_EXCH state in show crypto isakmp sa that never advances.

## 8.2 OSPF Missing Default Route at IKY Site

Problem: After full configuration, IKY core switches could not ping 8.8.8.8 or reach the internet despite HQ working correctly. The show ip route 0.0.0.0 command returned Network not in table on all IKY core switches.

Root Cause: NXR-IKY-EDGE-R1 was missing the default-information originate command under router ospf 1. This command advertises the default route to all OSPF neighbours. Without it, the IKY core switches had no gateway of last resort and dropped all non-specific traffic.

Resolution: Adding default-information originate to NXR-IKY-EDGE-R1 immediately propagated the default route as an OSPF External Type 2 entry to all IKY core switches. This was identified during configuration recovery after the device was inadvertently reset.

## 8.3 Stacked NAT Rules After Configuration Change

Problem: After modifying the NAT ACL to resolve the IPsec conflict, the original NAT rule was not removed before the new one was added. Both rules were active simultaneously — ACL 10 (original, permit any) and ACL 110 (new, with deny for tunnel endpoints). NAT matched ACL 10 first and continued translating IPsec traffic.

Resolution: The old rule was removed using no ip nat inside source list 10 interface GigabitEthernet5 overload. This required first clearing active NAT translations using clear ip nat translation * as IOS prevents removal of rules with active translations.

Key Lesson IOS does not replace NAT rules — it stacks them. Old rules must be explicitly removed. Always verify with show run ¦ include nat after making changes. clear ip nat translation * must be run before removing a rule with active translations. The ip nat inside tag must be applied to every LAN-facing interface, not just the primary one.

## 8.4 HSRP and STP Alignment

Challenge: Without deliberate configuration, both HSRP active gateway election and STP root bridge election are determined by default values — HSRP by highest IP address, STP by lowest MAC address. These two independent elections have no awareness of each other, frequently resulting in misalignment where the STP root is a different switch from the HSRP active gateway.

When misaligned, traffic from access switches follows the STP-preferred path to the root bridge, then traverses an additional hop to reach the HSRP active gateway. This creates suboptimal routing and wastes bandwidth on every frame for that VLAN.

Resolution: STP priorities were manually configured to ensure the HSRP active switch for each VLAN is also the STP root for that VLAN. This was verified by cross-referencing show standby brief and show spanning-tree vlan outputs on each core switch.

# 9. Verification and Testing

## 9.1 Routing Verification

The following commands confirm correct routing operation across both sites:

NXR-HQ-EDGE-R1# show ip route ospf

10.0.0.0/24 is subnetted, 1 subnets

O       10.1.1.0 [110/2] via 12.1.1.1, GigabitEthernet1

O       20.1.1.0 [110/2] via 12.1.1.1, GigabitEthernet1

O       30.1.1.0 [110/2] via 13.1.1.1, GigabitEthernet2

O       40.1.1.0 [110/2] via 13.1.1.1, GigabitEthernet2

NXR-IKY-EDGE-R1# show ip eigrp neighbors

H   Address          Interface   Hold  Uptime

0   192.168.10.1     Tu1           14   00:02:29

## 9.2 IPsec Verification

NXR-HQ-EDGE-R1# show crypto isakmp sa

dst        src        state      conn-id  status

2.2.2.2    1.1.1.1    QM_IDLE    1031     ACTIVE

QM_IDLE confirms Phase 1 is established. Packet counters in show crypto ipsec sa confirm active encryption of tunnel traffic.

## 9.3 HSRP Verification

NXR-IKY-CORE-SW1# show standby brief

Interface  Grp  Pri P  State   Active    Standby     Virtual IP

Vl50        20  110 P  Active  local     50.1.1.253  50.1.1.100

Vl60        20   90    Standby 60.1.1.253 unknown    60.1.1.100

## 9.4 NTP Verification

NXR-HQ-EDGE-R1# show ntp status

Clock is synchronized, stratum 2, reference is 8.8.8.8

clock offset is 30.0000 msec, loopfilter state is CTRL

## 9.5 End-to-End Connectivity Tests

| Test | Source | Destination | Result |
| --- | --- | --- | --- |
| Inter-VLAN (same site) | VLAN 10 (10.1.1.x) | VLAN 40 (40.1.1.x) | Pass |
| Cross-site VLAN | VLAN 10 (10.1.1.x) | VLAN 60 (60.1.1.x) | Pass |
| Internet access (HQ) | NXR-HQ-CORE-SW1 | 8.8.8.8 | Pass |
| Internet access (IKY) | NXR-IKY-CORE-SW1 | 8.8.8.8 | Pass |
| Cross-site from edge | NXR-HQ-EDGE-R1 | 60.1.1.1 | Pass |
| IPsec tunnel | NXR-HQ-EDGE-R1 | NXR-IKY-EDGE-R1 (192.168.10.2) | Pass |

# 10. Recommendations for Production Deployment

The following items were not implemented in this engagement due to simulation environment constraints but are strongly recommended for any production deployment of this design:

## 10.1 Security Enhancements

- Replace pre-shared key IPsec authentication with PKI certificate-based authentication for stronger identity verification

- Implement AAA (Authentication, Authorisation, Accounting) with a RADIUS or TACACS+ server to provide centralised device access control and full audit logging of all administrative actions

- Deploy a dedicated syslog server running rsyslog or Graylog to replace the current BusyBox-based collection — full indexing, alerting, and long-term retention are required for financial services compliance

- Configure SNMPv3 with authentication and encryption for device monitoring integration with a network management platform

## 10.2 Operational Improvements

- Deploy an automated configuration backup solution such as Oxidized or RANCID — the value of this was demonstrated during this engagement when a device was inadvertently reset and manual recovery was required

- Implement IP SLA with ICMP jitter probes in addition to reachability probes to detect WAN path degradation before it causes outages

- Configure SNMP traps to forward critical alerts to a network management system for proactive monitoring

- Establish a change management process with configuration version control before any production changes are made

## 10.3 Design Considerations

- Consider implementing VRF (Virtual Routing and Forwarding) to provide complete routing separation between Finance (VLAN 10) and other departments — appropriate for a financial services environment

- Evaluate BGP as a replacement for static routing at the WAN edge if the client acquires a second ISP connection for redundancy

- Review VLAN 200 that was found configured on NXR-IKY-ACC-SW2 during the engagement — this VLAN has no corresponding SVI or definition in the network design and should be investigated and removed

# Appendix A: Device Access Information

Security Notice Credentials documented here should be changed immediately upon production deployment. The pre-shared key LABKEY123 is a lab credential and must never be used in production. All passwords should be managed through a privileged access management (PAM) system.

| Device | Management Access | Username | Protocol |
| --- | --- | --- | --- |
| All devices | Console / VTY | netadmin | SSH v2 only |
| ISP-TRANSIT-R1 | Not in client scope | netadmin | SSH v2 only |
| IPsec Pre-Shared Key | LABKEY123 (REPLACE IN PRODUCTION) | - | - |

# Appendix B: Quick Reference — Key Show Commands

| Purpose | Command | Key Output to Look For |
| --- | --- | --- |
| IPsec Phase 1 status | show crypto isakmp sa | QM_IDLE = healthy |
| IPsec packet counters | show crypto ipsec sa | pkts encaps/decaps incrementing |
| OSPF neighbour status | show ip ospf neighbor | FULL state on all neighbours |
| EIGRP neighbour status | show ip eigrp neighbors | Tunnel1 neighbour present |
| HSRP gateway status | show standby brief | Active/Standby roles as designed |
| STP root verification | show spanning-tree vlan X | Root ID matches intended switch |
| NTP sync status | show ntp status | Clock is synchronized |
| IP SLA probe results | show ip sla statistics | Return code OK |
| Tracking object status | show track 1 | Reachability is Up |
| NAT translations | show ip nat translations | Inside local/global entries present |
| EtherChannel health | show etherchannel summary | P = bundled, SU = up |
| Route table check | show ip route | All expected networks present |